HETEROTYPER COMPUTATIONAL PIPELINE INSTRUCTIONS

Initial Notes:

All samples were run using MATLAB R2023b Update 1 (Version: 23.2.0.2380103).

For colony growth quantification of:

- Bright: Agar plate images of a total of 4 samples contributing to 4 sample groups (3h, 7h, 24h, and 48h) are provided under Data/Bright.

- Dark: Agar plate images of a total of 6 samples contributing to 3 sample groups (MLN, Spleen, and Liver) are provided under Data/Dark.

* Current folder pane in MATLAB must be navigated to the folder where the function file is when running each function. This is especially important for the main function “HeteroTyper_colony_growth_quantification.m”.

Usage:

- Save the folder containing images and MATLAB codes under your desired file path.

- Open MATLAB. Navigate to the folder containing the HeteroTyper pipeline (Bright or Dark). Add the folder and subfolders to the path.

- Open “HeteroTyper_colony_growth_quantification.m” and change metadata information with the current file path including file name and extension (*.xlsx).

- Run “inp = your_parent_image_folder_path;” in the terminal to assign the main folder path. This folder path must contain the subfolders with the images for each sample.

- Run “data = HeteroTyper_colony_growth_quantification(inp);” to start the image analysis. Analysis results are manually assigned to “data” parameter to save the generated data in a custom parameter. If not assigned, MATLAB will automatically save it in a parameter called “ans”. Custom assignment is crucial. Otherwise, after running each function under Code folder, the value of “ans” will change, unless a manual parameter name assigned.

- Saving MATLAB workspace containing the “data” after running the pipeline is recommended. Saved backup file can be imported by running “load(“file_path\backup_name.mat”);”

- For running the plot functions, please refer to the “Details of each function” section.

- For information on changing the parameters, please refer to the “Detailed information on adjustable parameters” section.

SECTION 1

DETAILES OF PLOT OR EXPORT FUNCTIONS

PLOT FUNCTIONS

- plot_debug_images.m

Generates diagnostic figures that summarizes the image-processing steps for each plate that passes the growth quantification criteria (minimum and maximum colony numbers on plates). For each time point/organ, it iterates over the corresponding plates and retrieves the debug images stored in data.processed{...}.colonies.debug. The function then plots a panel of eight images showing the original grayscale plate, background and background-flattened images, normalized, contrast-enhanced, and smoothed versions, the filtered binary mask, and a final overlay of the segmentation mask on a brightened grayscale image. These debug images provide a quality-control step to understand image processing.

Run following in the terminal to initialize the analysis;

plot_debug_images(data);

- plot_individual_plate.m

Provides an interactive view of a single plate and its colony-level growth dynamics, side-by-side. After specifying the plate position (1–104) and a color mode, the function overlays the final segmentation on the brightened grayscale plate image and labels all colonies that passed the filters. Depending on the selected mode (0, 1, 2), colonies are either shown with random colors (0), lag time (1), or final colony size (2). The function also plots the over-time size trajectories for all colonies on that plate, allowing you to directly link each colony’s spatial position to its growth curve and lag behavior.

Run following in the terminal to initialize the analysis;

plot_individual_image(data);

- plot_individual_colony.m

Generates images for selected individual colonies. After specifying the plate position (1–104) and a color number, the function overlays the final segmentation on the brightened grayscale colony image. Perimeter of the segmented colony is outlined with a green border. Colony growth parameters (final colony size, eccentricity, circularity, and solidity) are shown on the top-right corner of the generated image.

Run following in the terminal to initialize the analysis;

plot_individual_colony(data);

- plot_individual_colony.m

Generates images for selected individual colonies. After specifying the plate position (1–104) and a color number, the function overlays the final segmentation on the brightened grayscale colony image. Perimeter of the segmented colony is outlined with a green border. Colony growth parameters (lag time, final colony size, final colony intensity, intensity per size, eccentricity, circularity, and solidity) are shown on the top-right corner of the generated image.

Run following in the terminal to initialize the analysis;

plot_individual_colony(data);

- plot_QC_full_dataset.m

This function provides an overview quality-control (QC) check for the whole experiment. It first plots, for each plate, the total number of detected colonies versus those passing the colony filter and compares automated counts with manual counts using linear and log–log scale plots. Then, summarizes colony size distributions per plate as histograms, highlighting plates with very few or many colonies. It also creates image montages of segmented colonies (segmentation mask overlaid on the plate image) with colony counts annotated, enabling rapid visual inspection of segmentation quality across plates.

Run following in the terminal to initialize the analysis;

plot_QC_full_dataset(data);

- plot_QC_growth_data.m

This function provides an overview QC for the computed growth parameters. It summarizes colony growth over-time as line plots and size, lag time and early doubling time distributions as histograms per plate. Number of colonies are printed on each respective subplot per plate.

Run following in the terminal to initialize the analysis;

plot_QC_growth_data(data);

- plot_growth_data_across_sampletypes.m

Generates separate figures containing subplots of each sample individually to observe variances between samples/sample groups for the following.

Scatter plot:

- Final colony size vs. colony intensity

- Final colony size vs. lag time

- Final colony size vs. lag time (with density)

- Final colony size vs. early doubling time

- Final colony size vs. early doubling time (with density)

- Lag time vs. early doubling time

- Lag time vs. early doubling time (with density)

- Median values for final colony size, lag time, early doubling time, colony numbers

Histogram:

- Final colony size distribution

- Lag time distribution

- Early detection distribution

Line plot:

- Distributions of final colony size, lag time and early doubling time

Run following in the terminal to initialize the analysis;

plot_growth_data_across_sampletypes(data);

- plot_combined_samples.m

This code combines the growth measurements across all plates that passed the growth quantification for each sample group (time-points: 3h, 7h, 24h, 48h, or organ: MLN, Spleen, Liver). For each group, it applies size and eccentricity filters and flags the colonies as valid. Then, generates tables that contains the combined data across all the colonies in each sample group and plots the distributions for the following. The distributions are plotted using different colors for each sample group and median values (red stroke), allowing side-by-side comparison of growth features.

- Lag time

- Final colony size

- Colony area

- Final colony intensity

- Mean gray intensity

- Intensity per size

- Perimeter

- Circularity

- Eccentricity

- Solidity

This function also exports the combined lag time tables for each sample group to the MATLAB workspace for cumulative distribution function (CDF) analysis.

Run following in the terminal to initialize the analysis;

plot_combined_samples(data);

- plot_combined_samples_growth_curves.m

It selects all plates where growth parameters were quantified and where colony counts are within the range (10-650 for bright plates and 10-350 for dark plates). For each sample group (time points or organ type) and finds the colony size trajectories and applies final colony size and eccentricity filter, keeping only colonies above these thresholds. The time-lapse colony size information of colonies in the same sample group are then grouped together in a single table. Then, the plots are generated as line plots of colony size versus time (hours), where each colony is represented with an individual line and color.

Run following in the terminal to initialize the analysis;

plot_combined_samples_growth_curves(data);

- plot_lagTime_vs_morphology.m

This function compares the lag time to other morphologies parameters for each sample group individually, while combining the colonies corresponding to the same sample group. Lag time is compared against the following parameters. Scatter plots are generated as subplots for each comparison within each sample group, using separate colors for each sample group.

- Final colony size

- Perimeter

- Colony area

- Final colony intensity

- Mean gray intensity

- Intensity per size

- Circularity

- Eccentricity

- Solidity

Run following in the terminal to initialize the analysis;

plot_lagTime_vs_morphology(data);

- plot_doubling_time.m

This code generates colony doubling times using the final colony size values to find how long each colony needs to grow from their first appearance of ≥100 pixels to twice that size. For each sample group, it applies size and eccentricity filters, extracts doubling time of the colonies, and plots their distributions as histograms with each color representing a sample group and their median values as a line.

Run following in the terminal to initialize the analysis;

plot_doubling_time(data);

- plot_correlation_matrix.m

Generates correlation matrix for the following parameters to assess the relations between different growth parameters;

- Lag time

- Final colony size

- Final colony area

- Final colony intensity

- Mean gray intensity

- Intensity per size

- Perimeter

- Circularity

- Eccentricity

- Solidity

Run following in the terminal to initialize the analysis;

plot_correlation_matrix(data);

- plot_CDF_with_statistics.m

This function uses the combined lag time tables from all plates and generates empirical survival curves (1-CDF) for each sample group. It sorts the lag times, computes the survival function, while saving statistics to the data such as median, inter-quantile ranges, min/median/max lag time, and mean/max slope. Among these, min/median/max lag time and mean/max slope values are printed to the command window. The survival curves are then plotted as line plots with markers in separate figures using linear or logarithmic scale, with distinct colors for each sample group to visualize the heterogeneity in lag times. This function also performs pairwise Kolmogorov-Smirnov and log-rank tests between groups and prints the corresponding p-values to the command window.

Run plot_combined_samples(data) function initially, to export lag time values for each sample group to the MATLAB workspace.

Then, run the following commands to initialize the analysis:

For Bright-Plate Pipeline:

lag_times = {combined_lag_time_3h, combined_lag_time_7h, combined_lag_time_24h, combined_lag_time_48h};

group_labels = {'3h','7h','24h', ‘48h’};

colors = [0.161, 0.741, 0.584; 0.67, 0.74, 0.098; 0.941, 0.153, 0.42; 0.91, 0.627, 0.02];

plot_CDF_with_statistics(lag_times, group_labels, colors);

For Dark-Plate Pipeline:

lag_times = {combined_lag_time_MLN, combined_lag_time_Spleen, combined_lag_time_Liver};

group_labels = {'MLN','Spleen','Liver'};

colors = [0.035, 0.761, 0.624; 0.949, 0.494, 0.008; 0.541, 0.412, 0.831];

plot_CDF_with_statistics(lag_times, group_labels, colors);

EXPORT FUNCTIONS

- export_data_to_output_directory.m

This function compiles colony morphology and growth data from all plates into tables for downstream analysis (e.g., in R or python). Plates are grouped by growth time or organ type, and for each group the function collects per-colony lag time, final size, area, intensity, perimeter, circularity, eccentricity, solidity, and centroid coordinates, using the same size and eccentricity filters as in the other functions. It generates two combined tables: one with all basic morphology and growth parameters, and a second that additionally includes intensity per size since it requires another filter. Additional filtering is required to exclude infinite values that might be generated while calculating intensity per size. Both tables are saved as Excel files (*.xlsx) in the specified output directory and also exported to the MATLAB workspace and .mat files for later use.

Run following in the terminal to initialize the analysis;

export_data_to_output_directory(data);

- export_colony_images.m

This function exports colony images based on a lag time threshold to a user-defined folder on your computer. It initially applies size and eccentricity filters (as in the other functions) and then uses a lag time threshold to select colonies to export. By default, lag time cutoff is 38 hours, so colonies with lag times above this threshold are saved. This threshold can be adjusted to target earlier or later appearing colonies, or replaced by another parameter (e.g., intensity, intensity per size, circularity).

For each selected colony, it crops a 100 pixel (px) window around its center point (centroid), blends the contrast-enhanced grayscale version of the raw image with the segmentation mask, and overlays growth parameters such as lag time, final colony size, colony area, final colony intensity, mean intensity, intensity per size, perimeter, circularity, eccentricity, and solidity. These annotated colony images are saved as PNG files in the specified output folder with subfolders of sample groups and category-specific folder (e.g., Filtered or Successful colonies), with filenames as intensityPerSize_plateID_colonyID for easy downstream identification or use. Images are also stored in memory (not MATLAB workspace) for further inspection or figure generation.

Run following in the terminal to initialize the analysis;

export_colony_images(data);

- export_colony_img_intPerSize_threshold.m

This function exports colony images based on an intensity-per-size filter to a user-defined folder on your computer. It first groups plates by time point or organ (depending on pipeline used), then applies size and eccentricity thresholds together with lower and upper thresholds on intensity per size (by default lower: 25 and higher: 75) to select colonies either below or above the chosen range. This threshold can be adjusted or replaced by another parameter (e.g., intensity, intensity per size, circularity).

Colonies are cropped in 100 px windows around their centroid, shown as contrast-enhanced grayscale images with a green outline around the colony and a blended segmentation overlay. Growth parameters such as lag time, final colony size, colony area, final colony intensity, mean intensity, intensity per size, perimeter, circularity, eccentricity, and solidity are printed directly on each image. These annotated colony images are saved as PNG files in the specified output folder with subfolders of sample groups and category-specific folder (e.g., Filtered or Successful colonies), with filenames as intensityPerSize_plateID_colonyID for easy downstream identification or use.

Run following in the terminal to initialize the analysis;

export_colony_img_intPerSize_threshold(data);

SECTION 2

DETAILED INFORMATION ON ADJUSTABLE PARAMETERS

Chapter 1 – Main Code

File Name: HeteroTyper_colony_growth_quantification.m

- border_range (default: 0.85)

Fraction (%) of the plate radius used for analysis.

- Higher values (0.9–1.0): include more colonies near the plate edge, but also more edge artifacts.

- Lower values (0.7–0.8): more conservative, focus on central colonies and reduce edge effects.

- LoG_thresh (default: 0.09)

Intensity threshold used after Laplacian-of-Gaussian filtering to detect foreground (colony) pixels.

- Higher values: fewer, more conservative detections (only bright colonies).

- Lower values: more sensitive, may pick up faint colonies but also noise. Typical range: ~0.05–0.2 depending on imaging conditions.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

- lag_time_thresh (default: 100 px)

Colony size (in pixels) at which lag time is defined (i.e. when the colony first reaches this area).

  - Higher values (e.g. 150–200): lag time corresponds to a later, more robust growth stage.

  - Lower values (e.g. 50–80): lag time is measured earlier, but estimates become more sensitive to noise.

- early_DT_range (default: 5)

Fold-change used to compute early doubling time: DT = time(DT_range * lag_time_thresh) – time(lag_time_thresh).

  - Higher values (e.g. 6–8): measure doubling over a larger growth window (longer time interval).

  - Lower values (e.g. 3–4): focus on very early exponential expansion close to the lag threshold.

- min_colony_nr (default: 10)

Minimum number of “clean” colonies a plate must have to be included in growth quantification.

- max_colony_nr (default: 350 (Dark) or 650 (Bright))

Maximum number of “clean” colonies allowed per plate for growth quantification

  - Higher values or inf: include more densely populated plates, at the cost of potential segmentation errors and longer runtime.

  - Lower values (e.g. 250–300): skip very crowded plates where segmentation may be unreliable.

- plot_flag (default: 1)

Global plotting switch: enables/disables most plots in preprocessing and growth step.

  - 1 = plotting on (useful for debugging / method development

  - 0 = plotting off (faster batch processing)

- plot_flag_preprocessing (default: 0)

Controls plots related to background subtraction, cropping, and normalization.

  - Change to 1 to inspect raw vs processed images and adjust image preprocessing parameters.

  - Keep at 0 to run silently in large datasets.

- plot_flag_segmentation (default: 1)

Toggles visualization of colony detection and segmentation.

  - 1 is helpful to check colony masks and tune LoG_thresh, size_thresh, border_range, etc.

  - Set to 0 to speed up runs once parameters are fixed.

- plot_flag_growth (default: 1)

Controls plotting of colony growth curves and related summaries.

  - 1 = display individual / combined growth trajectories.

  - 0 = skip these plots for faster automated processing.

- data.metadata.original

The metadata for each plate is loaded from an .xlsx file using the template “data.metadata.original = readtable("file_path", "sheet", "sheet_name");“.

  - file_path should be replaced with the full path to the Excel metadata, including file extension, and sheet_name with the name of the worksheet that contains experimental annotations (e.g., sample_ID, growth time or organ, plate position).

Chapter 2 – Plate Center Detection

File Name: find_generic_plate_center.m

- nr_plates (default: 4)

Number of plate positions used in the experimental setup.

- Set this to the number of sample folders you want to use for center detection (e.g. if you have 24 plates, you should set it to 24 to use all of them, or a subset such as nr_plates = 8 (first 8 in alphabetical order!)).

- Subplot grid in subplot(4,6,i)

Controls how the diagnostic plate-center plots are arranged in the figure.

- The first two numbers (4, 6) correspond to number of rows (nrow) and columns (ncol) of subplots.

- You should choose them so that nrow * ncol ≥ number of plates (nr_plates). For example:

  - 24 plates → subplot(4,6,i) or subplot(3,8,i)

  - 10 plates → subplot(3,4,i)

- Adjust these values whenever you change nr_plates, so that all samples are displayed without overlapping or empty panels in unexpected places.

Chapter 3 – Colony Detection and Quantification of Growth Parameters

File Name: colony_detection_and_growth_quantification.m

- geom_props (features from regionprops)

geom_props is generated as a table. It defines which morphological and intensity based features are extracted for each colony from the segmented grayscale image via regionprops function. HeteroTyper includes:

- PixelIdxList

- Area

- Eccentricity

- Centroid

- Circularity

- ConvexArea

- ConvexHull

- EulerNumber

- MinorAxisLength

- MajorAxisLength

- Perimeter

- Solidity

- MaxIntensity

- MeanIntensity

- MinIntensity

If required, more fields can be added from regionprops function. Any parameter used later in the pipeline for exporting data or generating figures, must stay in this list while running the main code (HeteroTyper_colony_growth_quantification.m).

- b_radius (default: 0.85)

This parameter defines how much of the plate radius is considered valid for colonies when excluding border objects.

- Default is 0.85, meaning only the inner 85% of the plate is used for colony detection. Colonies touching or belonging to outer 15% are removed.

- Smaller values (e.g., 0.70 = 70%) are more aggressive border exclusion, keeps only colonies closer to the plate center.

- Larger values (e.g., 0.90), keeps much larger plate are, while risking agar cracks and segmentation errors.

- fig_w and fig_h (default: 1400 and 400)

These values define the width and height of the main figure that will be showing the debugging steps with plate images.

- If this figure size is large for your screen, reduce the values (e.g., fig_w = 800, fig_h = 200).

- Segmentation related parameters;

  - Background smoothing -> imgaussfilt(img_gray, X);

(Default: 30 (Bright) or 65 (Dark))

Here, the X is the standard deviation (σ, STD) of the Gaussian kernel used to estimate a very smooth background.

- Large STD (50-80) gives a slowly varying background that can ignore colonies and small features

- Lower STD (20-40) increases the background variation and leads to more noisy background

  - Gamma correction -> img_contrast = imadjust(img_norm, [], [], X);

(Default: 1 (Bright) or 1 (Dark))

Here, the X is the gamma value. It adjust the intensity of the image to improve contrast.

- Gamma = 1, no gamma correction

- Gamma < 1, brightens the mid-tones, helps to identify faint colonies, but amplifies the noise

- Gamma > 1, darkens the mid-tones, helps to suppress the background if bright colonies are present.

  - Fine smoothing -> img_smooth = imgaussfilt(img_contrast, X);

(Default: 1 (Bright) or 1.5 (Dark))

Here, the X is a small STD for fine smoothing before the thresholding.

- Lower STD (0.5-1), provides less smoothing, preserving fine details and more noise

- Higher STD (2-3), it applies stronger blur while reducing the noise and small identifications, but it can soften the colony edges or merge close colonies.

  - Minimal suppression for watershed -> D2 = imhmin(D, X);

(Default: 1 (Bright) or 1 (Dark))

Here, the X is the h-minima depth used before the watershed segmentation. imhmin(D, 1) suppresses shallow local minima in the distance transform D. A local minimum is a pixel whose value is lower than the surrounding pixels and therefore acts as a separate “basin” for the watershed algorithm.

- By increasing the h-value (e.g. from 1 to 2–3), only deeper minima (more pronounced basins) are kept, which reduces over-segmentation (too many tiny regions).

- Decreasing the h-value can help separate very close colonies but increases the risk of over-splitting.

Chapter 4 – Debug Images

File Name: plot_debug_images.m

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- fig_w and fig_h (default: 2000 and 800)

These values define the width and height of the main figure that will be showing the debugging steps with plate images.

- If this figure size is large for your screen, reduce the values (e.g., fig_w = 1200, fig_h = 600).

- rows and cols (default: 2 and 4)

These values define the layout of the debug figure. These values (rows x cols) provides 8 panels for plate images for each debugging step (gray, background, flat, normalized, smoothed, binary, segmented). If the number of images in the images list changes, you should adjust the number rows and/or columns accordingly.

Chapter 5 – Individual Plate Analysis

File Name: plot_individual_plate.m

- max_lag (default: 52 (Bright) or 55 (Dark))

This value defines the maximum lag time (in hours) a colony can have. In our case this lag time is based on the sum of room temperature incubation time and total imaging time. If an experiment runs for shorter/longer, this value must be adjusted accordingly.

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- fig_w and fig_h (default: 1200 and 400)

These values define the width (fig_w) and height (fig_h) of the main figure that will be showing the debugging steps with plate images.

- If this figure size is large/small for your screen, adjust the values.

Chapter 6 – Individual Colony Analysis

File Name: plot_individual_colony.m

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- crop_size (default: 150)

This parameter controls how big a region starting from the colony center is shown. This value can be decreased or increased to zoom in/out.

- text_lines (Overlayed text on colony images)

This parameter defines the values that will be printed on the colony image. Any parameter can be removed from the output by commenting it out with % (or deleting it), or add new lines that will display other parameters. The function will automatically write each value under the previous one.

- FontSize

The font size of the displayed growth features can be changed here: ‘FontSize’, 10

- TextColor

The font color of the displayed growth features can be changed here: ‘TextColor’, ‘white’

Change ‘white’ to any valid MATLAB color name (e.g., ‘yellow’, ‘green’).

Chapter 7 – Quality Control for Whole Data

File Name: plot_qc_full_dataset.m

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- runIx == 30 and ‘Size’, [5, 6]

These two parameters are linked.

- ‘Size’, [5, 6] means that the figure is drawn for 5 rows and 6 columns, making it to a total of 30 panels, which is the number of plates.

- runIx counts how many plate images you’ve collected in the current batch. runIx == 30 means, once you have images from 30 plates, it will draw a figure and reset the counter.

If you want a different grid size (e.g., 3x8), change the montage to ‘Size’, [3, 8], and change the batch size condition to runIx == 24.

- text_x and text_y

These two vectors give the x and y coordinates for the text labels, which is the number of colonies on the respective plates. This parameters are also connected to runIx and Size from previous bullet.

- [25:590:2800] creates position for 5 x-coordinates for the columns where the labels colony numbers should be printed.

- repmat(...,1,6) repeats each y-coordinate 6 times to get a 5x6 grid of coordinates that matches the 5x6 panels in the figure montage.

Chapter 8 – Quality Control for Growth Data

File Name: plot_qc_growth_data.m

- max_lag (default: 52 (Bright) or 55 (Dark))

This value defines the maximum lag time (in hours) a colony can have. In our case this lag time is based on the sum of room temperature incubation time and total imaging time. If an experiment runs for shorter/longer, this value must be adjusted accordingly.

- max_val (default: 4100 px (Bright) or 10800 px (Dark))

Maximum colony size value to define the colony size in plots.

  - This should be set after you run the function once and inspect the reported final colony sizes / plots.

  - Choose a value slightly above the largest observed colony size (e.g. max size + 100-500 px) so that the full distribution is captured without wasting any value far beyond the data range.

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

Chapter 9 – Growth Data Across Individual Samples

File Name: plot_growth_data_across_sampletypes.m

- n_groups and max_plate (default: 4 and 6)

These parameters will define the number of rows and columns of the generated figure depending on sample group size and the maximum plate number across all sample groups.

- n_groups represents the number of rows depending on the sample group number.

- max_plate represents the number columns depending on the maximum plate number

If you have less/more sample group or plates, these values must be adjusted accordingly.

- p_values_columns

This represents the column titles in the P-value matrix. Column names are the sample groups. If you have less/more sample groups, this label array must be defined accordingly.

- max_lag (default: 52 (Bright) or 55 (Dark))

This value defines the maximum lag time (in hours) a colony can have. In our case this lag time is based on the sum of room temperature incubation time and total imaging time. If an experiment runs for shorter/longer, this value must be adjusted accordingly.

- max_val (default: 4100 px (Bright) or 10800 px (Dark))

Maximum colony size value to define the colony size in plots.

  - This should be set after you run the function once and inspect the reported final colony sizes / plots.

  - Choose a value slightly above the largest observed colony size (e.g. max size + 100-500 px) so that the full distribution is captured without wasting any value far beyond the data range.

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- colors

This parameter represents a color matrix defining RGB colors ranging 0-1 for the different sample groups, in the same order as labels_type. If sample group number changes, the number of color entries must be changed, so that each group can have a distinct color.

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

Chapter 10 – Figures for Growth Parameters Across Combined Sample Group Data

File Name: plot_combined_samples.m

- max_lag (default: 52 (Bright) or 55 (Dark))

This value defines the maximum lag time (in hours) a colony can have. In our case this lag time is based on the sum of room temperature incubation time and total imaging time. If an experiment runs for shorter/longer, this value must be adjusted accordingly.

- max_val (default: 4100 px (Bright) or 10800 px (Dark))

- Maximum colony size value to define the colony size in plots.

  - This should be set after you run the function once and inspect the reported final colony sizes / plots.

  - Choose a value slightly above the largest observed colony size (e.g. max size + 100-500 px) so that the full distribution is captured without wasting any value far beyond the data range.

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- colors

This parameter represents a color matrix defining RGB colors ranging 0-1 for the different sample groups, in the same order as labels_type. If sample group number changes, the number of color entries must be changed, so that each group can have a distinct color.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- valid_idx_intS

In HeteroTyper pipeline, valid_idx represents the combined filter used to define the set of “valid” colonies. The colonies must be flagged as OK (col_flag = 1), larger than the chosen size threshold (col_size > size_threshold), and sufficiently round (col_ecc < ecc_threshold), and have finite intensity per size values (isfinite(ints_per_size)). This filter can be adjusted by changing size or eccentricity threshold or by replacing with another parameter (e.g., intensity, intensity per size, circularity).

Bin Size Value Definitions for Histograms

All xval* variables define the bin edges used for histcounts and subsequent bar plots. These variables should be tuned to the data range of the dataset used.

Important: For a new dataset, first run the pipeline once, check the minimum and maximum values for each parameter (lag time, size, intensity, etc.) and then update the xval_* ranges accordingly at the beginning of the code (including the plotting section).

- xval: 			Final colony size bins

- xval2: 			Lag time bins

- xval3: 			Early doubling time bins

- xval_area: 		Colony area bins

- xval_int: 		Colony intensity bins

- xval_mean_int:	Mean intensity bins

- xval_intSize:		Intensity per size bins

- xval_peri:		Perimeter bins

- xval_circ: 		Circularity bins (Range is always 0-1)

- xval_ecc: 		Eccentricity bins (Range is always 0-1)

- xval_sol: 		Solidity bins (Range is always 0-1)

Note: The same xval* ranges are also used later when constructing the plotting vectors (e.g. xval_plot1, xval_plot2, etc.). If you change the xval* definitions, make sure any corresponding plot vectors and axis limits remain consistent.

Chapter 11 – Growth Curves Across Combined Sample Group Data

File Name: plot_combined_samples_growth_curves.m

- max_lag (default: 52 (Bright) or 55 (Dark))

This value defines the maximum lag time (in hours) a colony can have. In our case this lag time is based on the sum of room temperature incubation time and total imaging time. If an experiment runs for shorter/longer, this value must be adjusted accordingly.

- max_val (default: 4100 px (Bright) or 10800 px (Dark))

- Maximum colony size value to define the colony size in plots.

  - This should be set after you run the function once and inspect the reported final colony sizes / plots.

  - Choose a value slightly above the largest observed colony size (e.g. max size + 100-500 px) so that the full distribution is captured without wasting any value far beyond the data range.

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- colors

This parameter represents a color matrix defining RGB colors ranging 0-1 for the different sample groups, in the same order as labels_type. If sample group number changes, the number of color entries must be changed, so that each group can have a distinct color.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- valid_idx

In HeteroTyper pipeline, valid_idx represents the combined filter used to define the set of “valid” colonies. The colonies must be flagged as OK (col_flag = 1), larger than the chosen size threshold (col_size > size_threshold), and sufficiently round (col_ecc < ecc_threshold). This filter can be adjusted by changing size or eccentricity threshold or by replacing with another parameter (e.g., intensity, intensity per size, circularity).

Chapter 12 – Comparison of Lag Times to Morphological Features

File Name: plot_lagTime_vs_morphology.m

- max_lag (default: 52 (Bright) or 55 (Dark))

This value defines the maximum lag time (in hours) a colony can have. In our case this lag time is based on the sum of room temperature incubation time and total imaging time. If an experiment runs for shorter/longer, this value must be adjusted accordingly.

- max_val (default: 4100 px (Bright) or 10800 px (Dark))

- Maximum colony size value to define the colony size in plots.

  - This should be set after you run the function once and inspect the reported final colony sizes / plots.

  - Choose a value slightly above the largest observed colony size (e.g. max size + 100-500 px) so that the full distribution is captured without wasting any value far beyond the data range.

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- colors

This parameter represents a color matrix defining RGB colors ranging 0-1 for the different sample groups, in the same order as labels_type. If sample group number changes, the number of color entries must be changed, so that each group can have a distinct color.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- valid_idx_intS

In HeteroTyper pipeline, valid_idx represents the combined filter used to define the set of “valid” colonies. The colonies must be flagged as OK (col_flag = 1), larger than the chosen size threshold (col_size > size_threshold), and sufficiently round (col_ecc < ecc_threshold), and have finite intensity per size values (isfinite(ints_per_size)). This filter can be adjusted by changing size or eccentricity threshold or by replacing with another parameter (e.g., intensity, intensity per size, circularity).

- param_names

This array list defines which growth features are plotted against lag time and in which order they will be displayed in the subplots. Each parameter will be displayed as the title of one column of plots. If you want to add or remove parameters, you must edit this list,  param_data, and y_limits.

- param_data

This list holds the data tables that are plotted against the lag time. The order and number of entries in param_data must match those in param_names. When adding/removing parameters, also update param_names and y_limits.

Chapter 13 – Doubling Time Comparison Across Combined Sample Group Data

File Name: plot_doublingTime.m

- max_lag (default: 52 (Bright) or 55 (Dark))

This value defines the maximum lag time (in hours) a colony can have. In our case this lag time is based on the sum of room temperature incubation time and total imaging time. If an experiment runs for shorter/longer, this value must be adjusted accordingly.

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- colors

This parameter represents a color matrix defining RGB colors ranging 0-1 for the different sample groups, in the same order as labels_type. If sample group number changes, the number of color entries must be changed, so that each group can have a distinct color.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- valid_colonies

In HeteroTyper pipeline, valid_colonies represents the combined filter used to define the set of “valid” colonies. The colonies must be flagged as OK (col_flag = 1), larger than the chosen size threshold (col_size > size_threshold), and sufficiently round (col_ecc < ecc_threshold). This filter can be adjusted by changing size or eccentricity threshold or by replacing with another parameter (e.g., intensity, intensity per size, circularity).

Chapter 14 – Correlation Matrix for Combined Sample Group Data

File Name: plot_correlation_matrix.m

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

- valid_idx_intS

In HeteroTyper pipeline, valid_idx represents the combined filter used to define the set of “valid” colonies. The colonies must be flagged as OK (col_flag = 1), larger than the chosen size threshold (col_size > size_threshold), and sufficiently round (col_ecc < ecc_threshold), and have finite intensity per size values (isfinite(ints_per_size)). This filter can be adjusted by changing size or eccentricity threshold or by replacing with another parameter (e.g., intensity, intensity per size, circularity).

- group_labels

An array containing the text labels for each sample group that will be printed in the figure. Sample group labels must be defined in the same other as lag_times.

  - group_labels = {‘3h’, ‘7h’, ‘24h’, ‘48h’};

  - group_labels = {‘MLN’, ‘Spleen’, ‘Liver’};

- corr_param_names

This parameter is a list that defines the title of the growth parameters that will be used in the correlation matrix. The number of parameter names must match the number of parameters used inside the param_matrix at the end of the function.

- Subplot grid in subplot(2,2,i)

The correlation matrices are arranged in a 2×2 grid, one subplot per sample group.

- 2,2 are the number of rows and columns in the figure. If you change the number of sample groups in ix{} / group_labels, you should adjust this layout according to:

n_rows × n_cols ≥ number_of_groups

- Use subplot(3,2,i) for 5–6 groups, subplot(1,3,i) for 3 groups.

Chapter 15 – Cumulative Distribution Function (CDF)

File Name: plot_CDF_with_statistics.m

- lag_times

An array containing combined lag time tables for each sample group (growth time or organ type). Before generating this array, plot_combined_samples function must be called to generate the combined lag time tables to the MATLAB workspace (e.g., combined_lag_time_MLN, combined_lag_time_3h, etc.). Then define:

- lag_times = {combined_lag_time_3h, combined_lag_time_7h, combined_lag_time_24h, combined_lag_time_48h};

- lag_times = {combined_lag_time_MLN, combined_lag_time_Spleen, combined_lag_time_Liver};

Number of data in lag_times and group_labels must match for CDF function to work.

- group_labels

An array containing the text labels for each sample group that will be printed in the figure. Sample group labels must be defined in the same other as lag_times.

  - group_labels = {‘3h’, ‘7h’, ‘24h’, ‘48h’};

  - group_labels = {‘MLN’, ‘Spleen’, ‘Liver’};

- colors

This parameter represents a color matrix defining RGB colors ranging 0-1 for the different sample groups, in the same order as labels_type. If sample group number changes, the number of color entries must be changed, so that each group can have a distinct color.

Chapter 16 – Exporting Data to Output Directory

File Name: export_data_to_output_directory.m

- output_dir

This is the folder path where all files (data tables, workspace (*.mat) files) are exported.  This path must be changed to an existing directory on your computer.

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

Chapter 17 – Exporting Colony Images with “Lag Time” Threshold

File Name: export_colony_images.m

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- crop_size (default: 150)

This parameter controls how big a region starting from the colony center is shown. This value can be decreased or increased to zoom in/out.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

- text_lines (Overlayed text on colony images)

This parameter defines the values that will be printed on the colony image. Any parameter can be removed from the output by commenting it out with % (or deleting it), or add new lines that will display other parameters. The function will automatically write each value under the previous one.

- lag_threshold (default: 38 hours (room-temperature incubation time included))

This value defines the minimum lag time a colony must have (in hours, including room-temperature incubation) to be exported. Only colonies with lag_time > lag_threshold are selected for exporting. This value can be adjusted to select early or late appearing colonies.

- target_colonies

This filter determines which colonies are exported. This filter can be modified to change the selection criteria (for example to select different lag time ranges).

In default setup, a colony must:

- be flagged as OK (col_flag)

- lag time above lag_threshold

- larger than size_threshold

- sufficiently round (col_ecc < ecc_threshold)

- file_path

This parameter defines the folder directory where colony images will be exported/saved. The function automatically creates subfolders for each sample group under this path. File path must be changed to an existing directory on your system.

- FontSize

The font size of the displayed growth features can be changed here: ‘FontSize’, 10

- TextColor

The font color of the displayed growth features can be changed here: ‘TextColor’, ‘white’

Change ‘white’ to any valid MATLAB color name (e.g., ‘yellow’, ‘green’).

Chapter 18 – Exporting Colony Images with “Intensity Per Size” Threshold

File Name: export_colony_img_intPerSize_threshold.m

- incTime (default: 20 (Bright) or 24 (Dark))

Room-temperature incubation time (in hours) that is added to the elapsed acquisition time.

- crop_size (default: 150)

This parameter controls how big a region starting from the colony center is shown. This value can be decreased or increased to zoom in/out.

- file_path

This parameter defines the folder directory where colony images will be exported/saved. The function automatically creates subfolders for validity of the colonies under this path. File path must be changed to an existing directory on your system.

- min_col and max_col (default: 10 and 650 (Bright) or 10 and 350 (Dark))

Define the allowed range of “valid” colonies per plate.

- Increase min_col to restrict analysis to plates with many colonies (better statistics).

- Decrease max_col to exclude very crowded plates; increase it to accept more dense plates (with possible segmentation challenges).

- Sample group definitions via metadata (default: ix_*)

Sample groups are currently defined using the Time or Organ column in the metadata. If you want to group by a different variable (e.g. strain, treatment, organ) or different time points, you must:

- Change the metadata column name (Time → e.g. Strain, Treatment, Organ, etc.).

- Update the values used for selection (e.g. '3H' → 'WT', 'Mutant', 'MLN', etc.).

- Adjust the corresponding index variables ix_* and the labels_type array to match the new groups.

- The rest of the function will then generate combined growth curves for these newly defined sample groups.

- size_thresh (default: 100 px (Bright) or 200 px (Dark))

Minimum colony size (pixels) to be considered for growth quantification.

- Higher values (e.g. 150–300): ignore very small / early colonies, focus on well-grown colonies.

- Lower values (e.g. 50–100): include more small colonies but also increase sensitivity to segmentation noise.

- eccentricity_thresh (default: 0.70)

Maximum allowed eccentricity for an object to be treated as a colony (0 = perfect circle, 1 = line).

- Lower values (0.5–0.6): keep mostly round colonies, filter out elongated/irregular objects.

- Higher values (0.8–0.9): keep more irregular colonies (risk of including artifacts).

- IntS_thr_L and IntS_thr_H (default: 25 and 75)

These parameters define the low and high cutoffs to filter the colonies according to their intensity per size.

- This can be adjusted after you inspect the intensity per size distributions and finding respective thresholds.

- valid_intSize

This filter select colonies whose intensity per size is either lower than the low intensity per size threshold (int_per_size ≤ IntS_thr_H) or higher than the high intensity per size threshold (int_per_size ≥ IntS_thr_H).

- valid_size_ecc

This filters the colonies whose size is above size_threshold (col_size ≥ size_threshold) or eccentricity is below eccentricity threshold (col_ecc ≤ ecc_threshold).

- category_sets

This is a list to define the valid or filtered out colonies in two different sets.

- ‘Filtered’ takes the colony ID’s which didn’t pass the size, eccentricity or intensity per size filters.

- ‘Valid colonies’ takes the colony ID’s which did pass the size, eccentricity or intensity per size filters.

category_sets helps to identify valid or invalid colonies and export all of them at the same time, but to different folder depending on their validity.

- text_lines (Overlayed text on colony images)

This parameter defines the values that will be printed on the colony image. Any parameter can be removed from the output by commenting it out with % (or deleting it), or add new lines that will display other parameters. The function will automatically write each value under the previous one.

- FontSize

The font size of the displayed growth features can be changed here: ‘FontSize’, 10

- TextColor

The font color of the displayed growth features can be changed here: ‘TextColor’, ‘white’

Change ‘white’ to any valid MATLAB color name (e.g., ‘yellow’, ‘green’).